OPTIMA traffic signal controller

integral MOVA 8 Test Specification

UCM 673514

| Authority | Name |
| --- | --- |
| Author | Iain Ross |
| Reviewed | Simon Winter |
| Approved | Rob Harding |

Table of Contents

1	Document Introduction	4

1.1	Purpose of Document	4

1.2	Scope of the Document	4

1.3	Notices	4

2	Optima iMOVA TRL Verification Testing	4

3	Optima iMOVA Functional Testing	5

3.1	Hardware Requirements	5

3.2	Software Requirements	5

3.3	Configuration and MOVA dataset Requirements	5

3.4	Initial Setup Verification	6

3.5	Optima Webpage Testing	6

3.5.1	Webpage Dataset Handling	6

3.5.1.1	Transferring a Dataset	6

3.5.1.2	Retrieving a Dataset	7

3.5.1.3	Deleting a Dataset	7

3.5.1.4	Operation Management with MOVA Settings	7

3.5.1.5	Error Count and Reset Error Count	8

3.5.2	Webpage Online Data	9

3.5.2.1	Detectors/Channels Status	9

3.5.2.2	Confirm Bits Status	10

3.5.2.3	Force Bits Status	10

3.5.2.4	Status Indicator Sliders	10

3.5.2.5	Status Indicator Counters	10

3.5.2.5.1	Warmup	11

3.5.2.5.2	Error Count	11

3.5.3	Webpage Errors	11

3.5.4	Webpage Active Dataset	11

3.5.5	Webpage TMA Logs	11

3.5.5.1	Download	12

3.5.5.2	TMA Logs Structure	12

3.5.6	Webpage SatFlow Logs	12

3.5.6.1	Export	12

3.5.6.2	SatFlow Logs Structure	12

3.5.7	Webpage MOVA Messages	13

3.5.7.1	Message Display	13

3.6	MOVA Tools Testing	14

3.6.1	Introduction	14

3.6.2	Dataset Management	14

3.6.2.1	Transfer Dataset	14

3.6.2.2	Retrieve Dataset	14

3.6.3	Junction Mimic Diagram	15

3.6.3.1	Junction Mimic Diagram Display Options	15

3.6.3.2	Junction Mimic Diagram Runtime Settings	15

3.6.4	MOVA Data Plan Switching	16

3.6.4.1	Overview	16

3.6.4.2	Data plan Monitoring (for testing purposes)	16

3.6.4.3	Time of Day Operation	16

3.6.4.4	Reversionary Plan	17

3.6.4.5	Data Plan Detector Triggering	17

3.6.4.6	Clock Change Behaviour: Changes < 1 Minute	17

3.6.4.7	Clock Change Behaviour: Changes > 1 Minute	17

3.6.4.8	Removing Time of Day information	18

3.7	MOVA Special Conditioning Testing	19

3.7.1	Output to User Variable	19

3.7.2	Output to Output Channel	19

3.7.3	Output to Detector	19

3.7.4	Power Cycle of Controller	19

3.7.5	Cross-Stream Detector Activation from Output Channels	20

3.8	Miscellaneous Tests	21

3.8.1	CRB Operation	21

3.8.2	Minor Configuration Change	21

3.8.3	Major Configuration Change	21

3.8.4	MOVA Emergency/Priority Links Operation	21

3.8.5	Connection to Controller Lost Warning	21

4	Document Control	22

4.1	Maintenance and Distribution	22

4.2	Amendment History	22

4.3	Abbreviations	22

4.4	Referenced Documents	23

4.5	Related Documents	23

# Document Introduction

## Purpose of Document

The purpose of this document is to define the TRL and functional tests required to prove the design of the Optima traffic signal controller integral MOVA software application, iMOVA.

## Scope of the Document

The tests prescribed by this document are intended to identify design defects on the iMOVA software application. The tests require a fully functioning Optima traffic signal controller and associated testing hardware, which will be detailed in the sections. Test files are required for some of the tests, held in UCM and referenced in the relevant sections as appropriate.

The testing is split into two parts. TRL kernel implementation verification and Telent functional testing. The iMOVA functional testing requires that the respective Optima software version being tested with iMOVA also be subjected to functional safety testing. This requirement is to ensure that when iMOVA is installed, controller safety is not compromised.

## Notices

Throughout this document there are notices warning of potential health and safety risks when working with the Optima if safe working procedures are not followed. There are three types of notice used:

Warning notices are intended to serve as a reminder to personnel to take care when undertaking testing, there may be a personal or environmental danger to be aware of or a risk to the equipment if the guidance is not followed.

Electrical Hazard notices serve as a reminder to personnel to verify safety precautions due to the risk of electric shock. It is expected that all work shall be undertaken in accordance with the Electricity at Work Regulations and BS7671.

Information notices are used to highlight points of interest that are not necessarily a safety hazard. These use the blue information sign shown here.

# Optima iMOVA TRL Verification Testing

This section should only be carried out if the MOVA kernel has changed. The MOVA implementation in the Optima is tested using a test document provided by TRL with test MOVA configurations.  The controller under test uses the test configurations specific to each MOVA test in the TRL test specification MOVA M8.0 Regression Test Plan and Script version 1. Note the document title and history says 1.0, but the header says version 1.8

A local PC runs MOVA Tools with a connection patch to allow the MOVA Tools Simulation to activate the controller detection to exercise MOVA. The implementation is tested by running the TRL MOVA Tools simulator in accordance with the instructions in the test document. The MOVA messages are logged on the local PC, each file is tagged with some header information as prescribed in the TRL test specification. These files are then sent to TRL for analysis, and the resulting approval letter is stored at UCM 503223. The appropriate section of the results sheet should be signed off at this time.

Note: If the user is not local, a remote desktop connection to the PC can be used along with some special override software running on the local PC and the test controller.

## Hardware and Software Requirements

This following are general requirements, if anything special is required for a particular test it will be listed under the test. No lamps, I/O switches or loop simulation devices need to be connected to the controller.

### Required hardware

- Networked Windows 7 (or later) PC.

- Networked Optima 3L Controller:

  - 8x LSC (24 logical phases, 72 lamp outputs).

  - 5x IO 16-16 cards (79 useable inputs/outputs).

  - 5x LDC (40 integral loop detectors).

### Required software:

- Latest MOVA Tools.

- MOVA tools patch to connect simulator to controller.

- Correct Optima 3 software including MOVA package.

#### Hardware Connection Patch for MOVA Tools Simulation Testing

For MOVA Tools simulator to exercise MOVA on a real controller, some files within MOVA Tools installation need to be added/modified/replaced. The files are stored in UCM xxxxxx as MOVA8Testing.zip. This archive also includes the necessary files for the Optima Outstation, which are not needed for this test spec. For controller testing the following files are should be copied to the MOVA Tools installation folder, e.g. C:\Program Files (x86)\MOVA\MOVA Tools:

| connection.xml | Set the IP address in this file to that of the controller under test. |
| --- | --- |
| Newtonsoft.Json.dll | Copy over only. |
| SignalControllerEmulator.dll_optima | Rename the original file “SignalControllerEmulator.dll” as “SignalControllerEmulator.dll_TRL” and paste the “_optima” version is as the original file name. |

When loading a MOVA Tools file for simulation, double click the controller object (circled in red in the figure below) to ensure that for Simulation Data, Controller Type is “External MOVA Controller”. The default is “PCMOVA”.

Figure 1 - Setting the Correct Controller Type for Simulation

#### Remote Operation of the Controller

The Optima 3 controller in the Warwick lab may be accessed using a suitable VPN on the user computer to allow a remote desktop connection to a PC in the lab that has the necessary software to conduct this testing. The computer needs to have the Override Tool installed which acts as a remote manual panel. The controller must have the LabUseOnly package installed which facilitates the use of the Override tool.

Warning: Do not leave the LabUseOnly package installed on a controller when finished.  It must NEVER be used on controllers in the field.

## Outline Test Procedure

The testing process varies for each test, but follows the general steps outlined below:

- Load the controller configuration onto the test controller and ensure that the controller is error free.

- If using the Override tool to access controller functions, ensure that the panel is in Normal, the Signals switch is ON, and no other manual panel buttons are activated. It is important to exit the Override tool with the panel in this state, so that MOVA Tools can use the same interface for the simulation data, whilst signals remain ON.

- Log in to the controller web interface, navigate to MOVA > Stream 1 and use the TRL web pages to delete the dataset and all dynamic data from the controller before continuing. Orange bars > Dataset Manager > Delete tab > Delete button.

- Using MOVA Tools, open the .mova file for the test being conducted.

- Open the simulation diagram and double click the controller object and ensure that Controller Type is “External MOVA Controller”. The default is “PCMOVA”.

- Connect to the test controller, using the IP of the test controller, with port 6030 for MOVA stream 1. Upload the dataset when prompted.

- The MOVA simulation length is found in the Data Outline window in MOVA Tools. Navigate to: Simulation Objects > Simulation Timing Data to view the Simulation Period in minutes. A typical simulation runs for 3 hours from 8am to 11am simulation time.

- In the Junction Mimic Diagram open the MOVA Messages window:

  - Ensure that Auto Scroll is selected.

  - Ensure that SF Show Sat Flow Messages is selected.

  - Click the Log Messages button to select a destination text file for the MOVA messages. This defaults to the same location as the MOVA Tools file containing the dataset and simulation data.

- Enable MOVA from Runtime Settings in the Junction Mimic Diagram window.

- Ensure that the controller is warming up, to 1+ number of stages before taking control. The CRB is set active by default and the configuration runs FT when not in MOVA so should warm up automatically.

- Load the MOVA Tool simulator intervention file if one is required. This can pause the simulation after a fixed number of seconds, to allow for changing some detection for example.

Important! Intervention files must be loaded and MOVA message logging started prior to commencing the simulation run.

- Start MOVA Tools Simulator with the Play button at the bottom of the Simulation Diagram window.

Important! Ensure that the simulation speed is changed to x1 from the default x3.

- Perform any other actions during the test as required by the TRL test specification.

- At the end of the simulation, save the Delay/Count/Green statistics for all periods. The default folder is the location of the .mova file being used.

- Deselect the Log Messages button to stop logging MOVA messages. They will be saved in the log file specified in step 8.c.

[continued]

- Append the log file with the following header and save.

Test using	MOVA Tools: <version number>

Test number:		<TRL Regression Test Specification Test number>

MOVA version:		<Version from Optima webpage>

MOVA data file name:	<.mova filename>

Date of test:		<date in DD/MM/YYYY format>

- Disable MOVA via the Runtime Settings.

- Disconnect MOVA Tools from the controller.

## Kernel Implementation Required Tests

### Overview

The following tests are to be performed:

- Section 5.1: 01_Thurnby

- Section 5.2: 02_BigSim

- Section 5.2: 02_BigSim+Priority

- Section 5.3: 03_HugeMOVA

- Section 5.4: 04_Harlington_1

- Section 5.6: 06_Parkway_

- Section 5.7: SimDRT_

- Section 5.8: 8_Ped_Priority

- Section 5.10: Bonus_Active

- Section 5.10: Bonus_Inactive

- Section 5.13: 13_Candet

- Section 5.14: Harlington_PFACIL_a

- Section 5.14: Harlington_PFACIL_b

- Section 5.14: Harlington_PFACIL_c

- Section 5.15: SD_CODE_2

- Section 5.16: CMOVA_Bonus

- Section 5.18: 18_ShortCycles

- Section 5.19: 1_priority_stgs.txt

- Section 5.19: 2_priority_stgs.txt

### Test 1: TRL Test Section 5.1 - 01_Thurnby

#### Overview

Refer to section 5.1 of the TRL regression test specification. This is a simple junction which is oversaturated most of the time, but not all. Tests capacity maximisation and end-sat decisions.

Simulation length: 	180 minutes.

Required files:		Thurnby_0.4.cbn3

Thurnby_.mova

#### Procedure

- Load the configuration and ensure that the controller is running in Normal FT, with Signals ON and no errors.

- Delete the current dataset and all dynamic data using the controller webpages.

- Load the .mova file in MOVA Tools.

- Check the Simulation is using External MOVA Controller and not the default PCMOVA.

- Connect to the controller and download the dataset to the controller.

- Open the MOVA messages, enable SF messages and start logging to file.

- Enable MOVA and allow to warm up and take control.

- When in control, start the MOVA simulation at x1 speed (no intervention file required).

- During the simulation, periodically check there are no errors generated, if there are, the problem will need to be rectified and the simulation re-run.

- When the simulation has ended, stop logging the MOVA messages and save the log to a text file with the appropriate header.

- Save the Delay, Count and Green statistics for the simulation just run, when prompted to view them at the end of the simulation. They are saved as CSV file.

### Test 2: TRL Test Section 5.2 - 02_Bigsim

#### Overview

Refer to section 5.2 of the TRL regression test specification. This is general exercising of the normal functionality code with a fairly complex junction. Goes through capacity maximisation and stop and delay optimisation and stages very variable in length. The version with priority (see next test) exercises priority code including cancel detectors.

Simulation length: 	180 minutes

Required files:		Bigsim_0.3.cbn3

Bigsim_. Mova

#### Procedure

- Read the outline test procedure in section 2.2 and follow the steps in Test 1.

### Test 3: TRL Test Section 5.2 - 02_Bigsim+Priority

#### Overview

Refer to section 5.2 of the TRL regression test specification. This is general exercising of the normal functionality code with a fairly complex junction. Goes through capacity maximisation and stop and delay optimisation and stages very variable in length. The version with priority (this test) exercises priority code including cancel detectors.

Simulation length: 	125 minutes

Required files:		Bigsim_0.3.cbn3

Bigsim_. Mova

#### Procedure

- Read the outline test procedure in section 2.2 and follow the steps in Test 1.

### Test 4: TRL Test Section 5.3 - 03_HugeMOVA

#### Overview

Very large site with 21 lanes, 31 links and 10 stages. It also has two priority links. Stretches MOVA to its limits and runs both congested and uncongested.

Simulation length: 	60 minutes

Required files:		HugeSim_0.3.cbn3

HugeMOVA. mova

#### Procedure

- Read the outline test procedure in section 2.2 and follow the steps in Test 1.

### Test 5: TRL Test Section 5.4 - 04_Harlington_1

#### Overview

Test including standard bus priority. The TRL test specification instructs that neither BUSMAX nor BUSWGT are in use, but a check of the priority settings shows that neither are enabled in the version of the MOVA Tools file Telent possess.

Simulation length: 	180 minutes

Required files:		Harlington_0.2.cbn3

Harlington_1_.mova

#### Procedure

- Read the outline test procedure in section 2.2 and follow the steps in Test 1.

### Test 6: TRL Test Section 5.6 - 06_Parkway

#### Overview

Test of a “real-life” junction with normal MOVA functionality. Has left turn filters and is slightly different from the previous version of Parkway.

Simulation length: 	60 minutes

Required files:		Parkway_0.2.cbn3

Parkway_+confs_MT2.mova

#### Procedure

- Read the outline test procedure in section 2.2 and follow the steps in Test 1.

### Test 7: TRL Test Section 5.7 - 07_SimDRT

#### Overview

Test 6MRR sequence for double-right-turn situations, adjusting the flows to test the stage sequence changes according to flows. Up to 122 minutes there is occasional running of the stage sequence 1,3,4,5. After that, there is less running of stage 3 in favour of stage 4. MOVA Tools allows the flows to be varied from within the data to test the different stage sequence possibilities, so there are no special instructions.

Note: The original Optima 2 config had an erroneous termination type 2 on phase B. This caused the simulation to end prematurely with an intergreen failed to end error in MOVA due to an illegal stage move. This has been removed in Optima 3.

Simulation length: 	300 minutes

Required files:		SimDRT_0.4.cbn3

SimDRT_.mova

#### Procedure

- Read the outline test procedure in section 2.2 and follow the steps in Test 1.

### Test 8: TRL Test Section 5.8 - 08_Ped_Priority

#### Overview

This test is designed to exercise the PEDMAX facility. The reason for increasing the flows is to change the simulation junction to become oversaturated and for reducing them is to show that the maximums are maximums and that MOVA can change before they are reached if appropriate. Flows are adjusted within the simulation so there are no special instructions.

Simulation length: 	180 minutes

Required files:		PedPriority_0.3.cbn3

Ped_Pri_MT2_.mova

#### Procedure

- Read the outline test procedure in section 2.2 and follow the steps in Test 1.

### Test 9: TRL Test Section 5.10 - 10_Bonuses

#### Overview

There are two tests to run that test the bonus code. The two tests are the same but the first has the links set so that the cycle time isn’t cut, and the second has all three links set to cut the cycle time if the bonuses are high enough. Flows are adjusted within the simulation so there are no special instructions.

Simulation length: 	60 minutes each

Required files:		Bonus_0.2.cbn3

Bonus_Inactive_.mova

Bonus_Active_.mova

#### Procedure

- Read the outline test procedure in section 2.2 and follow the steps in Test 1.

- Delete the dataset and all dynamic data using the controller web interface.

- Close Bonus_Inactive.mova in MOVA Tools.

- Load the second Bonus_Active.mova in MOVA Tools.

- Follow the steps from step 4 onwards in Test 1.

Note: Save the MOVA message logs and Delay/Count/Green Stats with the name of the dataset used to allow easy identification later.

### Test 10: TRL Test Section 5.13 - 13_Candet

#### Overview

This tests the cancel detector function. All three CANDET options are tested. There are no special instructions.

In the MOVA dataset all stage-to-stage moves are allowed. Phases B and I appear in Stage 3 and are set in the configuration to terminate when phase A loses ROW. Phase A appears in stages 2 and 3 so a 3 to 2 move cannot complete without the controller locking up.  The original Optima 2 configuration had a stage 3 to 2 move prohibited in all modes, however with this programming, MOVA would fail with stage not ended or intergreen too long errors as the controller would wait in Stage 3 until another stage was requested. The Optima 3 configuration has been changed so that a 3 to 2 move is made via the All-Red stage. As the All-Red stage does not have a stage confirm, MOVA doesn’t see it and the 3 to 2 movement will complete as normal.

Figure 2 – MOVA Stage Diagram for 13_Candet.mova

Simulation length: 	185 minutes

Required files:		Candet_0.2.cbn3

13_Candet.mova

#### Procedure

- Read the outline test procedure in section 2.2 and follow the steps in Test 1.

### Test 11: TRL Test Section 5.14 - 14_PFACIL

#### Overview

This tests the PFACIL codes not covered in 04_Harlington_1 and 05_Harlington_2. This requires three additional test cases included in this folder. The sites in this test are the same as the Harlington data above, with different PFACIL codes.

This also tests that using the codes (same codes for all priority links) gives the same answer as MOVA M7.

Simulation length: 	180 minutes each

Required files:		Harlington_0.2.cbn3

Harlington_PFACIL_a.mova

Harlington_PFACIL_b.mova

Harlington_PFACIL_c.mova

#### Procedure

- Read the outline test procedure in section 2.2 and follow the steps in Test 1.

- Delete the dataset and all dynamic data using the controller web interface.

- Close Harlington_PFACIL_a.mova in MOVA Tools.

- Load the second Harlington_PFACIL_b.mova in MOVA Tools.

- Follow the steps from step 4 onwards in Test 1.

- Delete the dataset and all dynamic data using the controller web interface.

- Close Harlington_PFACIL_b.mova in MOVA Tools.

- Load the second Harlington_PFACIL_c.mova in MOVA Tools.

- Follow the steps from step 4 onwards in Test 1.

Note: Save the MOVA message logs and Delay/Count/Green Stats with the name of the dataset used to allow easy identification later.

### Test 12: TRL Test Section 5.15 -15_SDCODE_2

#### Overview

This tests stage demand code 2. The test uses the Bushy Road (Merton) data with four pedestrian link SD codes changed to 2. There are no special instructions.

Simulation length: 	60 minutes

Required files:		BusheyRd2_0.5.cbn3

BusheyRd2.mova

#### Procedure

- Read the outline test procedure in section 2.2 and follow the steps in Test 1.

### Test 13: TRL Test Section 5.16 -16_CMOVA_bonus

#### Overview

This tests the workings of the bonus code in Compact MOVA. Some cycles should be cut short, see the Cut flag in the messages. There are no special instructions.

Simulation length: 	60 minutes

Required files:		CMOVA_Bonus.0.2.cbn3

CMOVA_Bonus.mova

#### Procedure

- Read the outline test procedure in section 2.2 and follow the steps in Test 1.

### Test 14: TRL Test Section 5.18 - 18_ShortCycles

#### Overview

This tests MOVA with a low TOTALG value of 50 seconds. There are no special instructions.

Simulation length: 	125 minutes

Required files:		ShortCycles_0.3.cbn3

ShortCycles_.mova

#### Procedure

- Read the outline test procedure in section 2.2 and follow the steps in Test 1.

### Test 15: TRL Test Section 5.19 - 19_e-p_demand_crash

#### Overview

In MOVA version M6 and early versions of M7 a bug was introduced whereby if control returned to the main stage after an emergency/priority call, MOVA would remain in that stage after getting trapped in an infinite loop. This test is to confirm that the bug is no longer there. There are two datasets to run, each with their own intervention file to set detector states.

Simulation length: 	60 minutes each.

Required files:		demandCrash_0.2.cbn3

Run 1_priority_stgs.mova

Test_1_priority_stgs.ivc

Run 2_priority_stgs.mova

Test_2_priority_stgs.ivc

#### Procedure Test 1

- Read the outline test procedure in section 2.2 and follow the steps below:

- Load the configuration and ensure that the controller is running in Normal FT, with Signals ON and no errors.

- Delete the current dataset and all dynamic data using the controller webpages.

- Load the Run 1_priority_stgs.mova file in MOVA Tools.

- Check the Simulation is using External MOVA Controller and not the default PCMOVA.

- Connect to the controller and download the dataset to the controller.

- Open the MOVA messages, enable SF messages and start logging to file.

- Load the intervention file Test_1_priority_stgs.ivc in the simulation window.

- Enable MOVA and allow to warm up and take control.

- When in control, start the MOVA simulation at x1 speed.

- During the simulation, periodically check there are no errors generated, if there are, the problem will need to be rectified and the simulation re-run.

- When the simulation has ended, stop logging the MOVA messages and save the log to a text file with the appropriate header.

- Save the Delay, Count and Green statistics for the simulation just run, when prompted to view them at the end of the simulation. They are saved as CSV file.

#### Procedure Test 2

- Read the outline test procedure in section 2.2 and follow the steps below:

- Load the configuration and ensure that the controller is running in Normal FT, with Signals ON and no errors.

- Delete the current dataset and all dynamic data using the controller webpages.

- Load the Run 2_priority_stgs.mova file in MOVA Tools.

- Check the Simulation is using External MOVA Controller and not the default PCMOVA.

- Connect to the controller and download the dataset to the controller.

- Open the MOVA messages, enable SF messages and start logging to file.

- Load the intervention file Test_2_priority_stgs.ivc in the simulation window.

- Enable MOVA and allow to warm up and take control.

- When in control, start the MOVA simulation at x1 speed.

- During the simulation, periodically check there are no errors generated, if there are, the problem will need to be rectified and the simulation re-run.

- When the simulation has ended, stop logging the MOVA messages and save the log to a text file with the appropriate header.

- Save the Delay, Count and Green statistics for the simulation just run, when prompted to view them at the end of the simulation. They are saved as CSV file.

Note: This concludes the kernel testing. Gather up the MOVA Message log files for each test and compress to a zip file. Send this to TRL for analysis and approval.

# Optima iMOVA Functional Testing

Important: Some tests can be completed while testing other parts of the specification. For example, during the webpage Dataset Handling testing, it is possible to verify operation of the Detector/Confirm/Force bit status. Browse the results sheet prior to commencing the testing with this in mind.

The MOVA functional tests detailed in this document should be recorded in spreadsheet Optima iMOVA Test Results, UCM: 672664.

## Hardware Requirements

A PC connected via Ethernet to a 16-phase ELV rack with 48 loop detectors and 2 I/O 16-16 Cards is required. For testing, the 16 phases should be connected to dummy loads for simulating lamp faults and conflicts and measuring response times as part of the safety testing. The detector cards need to be either dummy switch cards or standard 4-channel cards connected to suitable dummy loops. The I/O requires a 16-input switch box, and 32 outputs can be connected if the I/O card output LEDs are not visible during the testing.

## Software Requirements

The controller should be loaded with the relevant core software and iMOVA application software under test.

Package manager version 3.1 will be required to load the configurations to the controller. A USB stick can be used if package manager is unavailable. Optima Configurator software will be required on the PC.

## Configuration and MOVA dataset Requirements

There are six configurations and datasets used for testing, held in UCM 673512 detailed below:

| Ref. | Configuration | Dataset | Description |
| --- | --- | --- | --- |
| A | 4STRMOVAGB Iss 1.2 | 4 Stream Test - Conditioning RevertDP3.mova 4 Stream Test - Conditioning RevertDP3-OC5-8.mova | 4 stream using G bits and phase confirms, includes 4 data plans which can be switched by timetable or MOVA detector. MOVA Output Channels and detectors are used to show which data plan is in use via MOVA Special Conditioning. Also has special conditioning to provoke “wrong stage returned” errors. |
| B | 4STRMOVADS Iss 1.2 | 4 Stream Test - Conditioning RevertDP3.mova 4 Stream Test - Conditioning RevertDP3-OC5-8.mova | 4 stream using phase/stage direct, includes 4 data plans which can be switched by timetable or MOVA detector. MOVA Output Channels and detectors are used to show which data plan is in use via MOVA Special Conditioning. Also has special conditioning to provoke “wrong stage returned” errors. |
| C | CE448 Iss 4 | TS-113B.mova | Dual Stream configuration with MOVA linking and a BRIDGE input to switch between two data plans |
| Ref. | Configuration | Dataset | Description |
| D | NE002 Iss 4 | 327L+mc.mova | Single Puffin with MOVA priority links. 4 data plans switched by timetable or MOVA detector. Output channels used to show data plan running. |
| E | SE0241 Iss 6 | Regents Park test.mova | 8 stage single stream configuration with bus priority links and 2 data plans. |
| F | 130L-PD Iss 0.3 | No dataset required | Config with greater than 32 phases (including dummy and clone) and phase direct confirms. |

## Initial Setup Verification

Uses configuration A

- Load configuration A and install related dataset (via MOVA webpage or MOVA Tools). Navigate to Software | Versions to verify the versions and package numbers are correct.

- Navigate to Software | Licences and verify all four streams of MOVA are licenced or a valid temporary licence exists.

- Navigate to the first MOVA stream: MOVA | Stream 1. This shows the TRL webpages Online Data page. Verify the kernel version is correct for the version of MOVA under test.

- Repeat step 3 for the remaining three streams of MOVA.

- Record the kernel version on the results sheet, UCM 672664.

## Optima Webpage Testing

Tip: Consider running the MOVA Tools testing in parallel with the webpage testing to minimise repetition.

This section tests the Telent implementation of the TRL MOVA webpages. Where a reference to TRL menu is made, this is the 3 orange bars as shown below:

Figure 3 – TRL Webpage Menu

### Webpage Dataset Handling

Uses configurations A,B,C,D and E

Note: This section should be repeated using a range of configurations and datasets.

Before continuing, create MXDS files by exporting the .mova files from MOVA Tools. MXDS file format is required for the transferring datasets into the controller via the webpages.

#### Transferring a Dataset

- Load the configuration under test, switch signals ON and verify there are no errors on the controller.

- Open the controller webpages and navigate to the first MOVA stream: MOVA | Stream 1. The webpages should display a message stating Stream not available so perform step 4.

- If the online data page appears, then there is a dataset loaded. Delete the dataset, see section 3.5.1.3.

- Use the TRL menu to open the Dataset Manager page. Click the Transfer tab and then Select File button. Select Controller 1 from the drop-down list and click Transfer. Click Continue on the Remote Transfer warning message. Click Yes to the Operations Management warning if displayed. The stream should update the status underneath with Transferring dataset …. and Data File sent -awaiting load and finally Dataset loaded successfully.

- Verify the stream is displaying live information on the Online Data page. Since MOVA is not enabled, no Force bits status will be shown, but the Detectors/Channels status and Confirm bits status should be updating in real time. The Online Data page is the default view when a dataset exists, and the user navigates to MOVA | Stream 1. Alternatively use the TRL menu to select it.

- Repeat steps 1 to 4 for the remaining three streams.

- Check that the transfer can be aborted on each stream following the instructions at step 3 but choosing Abort on the Remote Transfer warning message and again by choosing No to the Operations Management warning message.

Note: It is good practice to delete a dataset before transferring a new dataset as this will remove any dynamic data stored with it and prevent any unexpected behaviour.

#### Retrieving a Dataset

- Navigate to the first MOVA stream: MOVA | Stream 1. The webpages will display the online data for the stream.

- Use the TRL menu to open the Dataset Manager page. Click the Retrieve tab and then Retrieve button. The dataset should be downloaded as an .MXDS file.

- Open the dataset with MOVA Tools and compare with the original MOVA Tools version to ensure no corruption has occurred.

#### Deleting a Dataset

- Navigate to the first MOVA stream: MOVA | Stream 1. The webpages will display the online data for the stream.

- Use the TRL menu to open the Dataset Manager page. Click the Delete tab and then Delete button to delete the dataset and all working / dynamic data.

- Verify it is possible to cancel the deletion by clicking No on the Transfer Management warning.

- Repeat step 2 and click Yes on the Transfer Management warning message. The stream should update the status underneath with Data deleted and MOVA restarting…

- Click on the first MOVA stream MOVA | Stream 1 again and the Dataset Manager page should open on the Transfer tab.

- Repeat steps 1 to 3 for the remaining streams.

#### Operation Management with MOVA Settings

- Navigate to the first MOVA stream: MOVA | Stream 1. The webpages will display the online data for the stream.

- Use the TRL menu to open the Dataset Manager page. Click the Settings tab and toggle the MOVA enabled button to ON. The button should change to ON and a Transfer Management confirmation message will appear. Click No and the button should return OFF.

- Repeat step 2 but click Yes to the confirmation message.  The button should change to ON.

- Return to the Online Data page and the MOVA enabled flag should also be ON.

- Use the TRL menu to open the Dataset Manager page. Click the Settings tab and toggle the MOVA enabled button to OFF. The button should change to OFF, and a Transfer Management confirmation message will appear. Click No and the button should return to ON. Verify the status of the MOVA enabled flag is still ON.

- Repeat step 5 but this time click Yes, and the button should stay OFF. Verify MOVA enabled flag is OFF on the Online Data page.

- Repeat step 2.

- Toggle the controller from Normal mode to Manual mode or toggle the detectors to generate demands to cycle the controller to warm up, depending on the configuration used the number of warmups will vary and is one more than the total number of stages. Verify after warmup MOVA assumes control of the stream (F-bits and TO will be seen active on the Online Data page).

- With MOVA in control of the stream, toggle the On-control button to OFF. Click No in the confirmation message to cancel the change.

- Toggle the On-control button to OFF again and click Yes to the confirmation message.

- Verify MOVA drops control of the stream and warmup value returns to 0.

- Cycle the controller stages and verify MOVA assumes control of the stream again after warmup.

- Repeat steps 1-12 for remaining streams.

#### Error Count and Reset Error Count

Uses configurations A and B.

Note: It is worthwhile checking operation in MOVA Tools (Section 3.6.3) in parallel with webpage functionality.

MOVA will error if the wrong stage is confirmed in response to it forcing a stage. To facilitate this, some special conditioning has been set up in configurations A and B. When in stage 1, if SCB 1 (for stream 1) active will inhibit stage 2, inhibit F2 and override F3 active. MOVA does not know about this. Inserting a suitable demand for stage 2 when in stage 1 will cause a move to stage 3 with SCB/1=1. The SCB can be cancelled once MOVA has errored and released control. The error will increment the Error count value by 5 in the connected MOVA stream.

- Load the configuration and related dataset to be tested. Switch signals ON, enable MOVA and toggle the detection to allow MOVA to take control.

- Verify stages 1 to 4 in the stream under test operate as intended under MOVA by toggling the detection for the correct phases and observing the Online Data webpage.

- Use the detection to drive the controller to stage 1 and rest there (no other detectors active) Toggle SCB/<stream no.> to initiate the special conditioning. For example, in stream 1 set SCB/1=1.

- Toggle detection for stage 2 and verify MOVA setting F2. The controller will move to stage 3 and reply confirm for 3. Verify MOVA immediately releases control:

  - F2,HI and TO are set inactive,

  - MOVA enabled remains ON,

  - On-control changes to OFF,

  - CRB remains ON,

  - Warmup remains number of stages + 1,

  - Error count increases by 5.

- Reset the SCB to 0 to cancel the conditioning. The controller will move from stage 3 to stage 2 in VA.

- After a few minutes MOVA will try to run again and will automatically change the On-control flag to ON, however this process can be bypassed with a manual intervention to set On-control back to ON. Verify MOVA has re-assumed control.

- Repeat steps 3 to 6 until the Error count reaches 20, and when this happens verify:

  - F2, HI and TO are set inactive,

  - MOVA enabled remains ON for around 10-15 seconds then changes to OFF,

  - On-control changes to OFF,

  - CRB remains ON,

  - Warmup remains number of stages + 1,

  - Error count increases to 20.

- Use the TRL menu to navigate to the Errors page to view the error log, which should look like the figure below:

Figure 4 – Example MOVA Error Log

- Verify the log, pressing refresh if required.

- Use the TRL menu to navigate to the Dataset Manager |Settings tab and press the Reset Error Count button. There is no confirmation for this action, so return to Online Data to verify the error count is now zero.

- Repeat steps 1 to 8 for remaining streams remembering to increment the SCB number set to toggle the special conditioning.

- Re-enable MOVA and set the On-control flag back to ON. Return to Online Data to verify the stream is now running MOVA again.

- Repeat for remaining stages, using SCBs 2, 3 and 4 respectively.

Repeat the tests 3.5.1.1 to 3.5.1.4 with a range of datasets to ensure a thorough test. Test 3.5.1.5 can only be undertaken using configurations A and B.

Once Section 3.5.1 has been successfully completed sign off the appropriate sections of the test document.

### Webpage Online Data

Uses configurations A,B,C,D, E and F.

Note: This section should be repeated using a range of configurations and datasets.

Unless this section is being tested independently, most of this section will have been covered while testing section 3.5.1, Dataset Handling.

#### Detectors/Channels Status

- Load the configuration and dataset, enable MOVA and cycle the controller to allow MOVA to take control.

- Navigate to the Optima webpage Online Data for the first stream, MOVA | Stream 1.

- Toggle a range of detectors and verify their status is correctly reported.

- Repeat for remaining streams of the configuration being used.

#### Confirm Bits Status

Note: Configuration A uses G-bit confirms, Configuration B is identical to A, except that it uses Stage/Phase Direct. All other configurations use phase/stage direct.

- Check that stage and phase confirms bits are correctly reported, across all stages and phases configured, for all streams running MOVA in the configuration under test.

- Ensure that configuration F has been tested as this has > 32 phase confirms into MOVA.

#### Force Bits Status

“Force bits” in this context are stage force Fn (n = stage number), Hurry Inhibit HI and Take Over TO.

- Switch signals ON and ensure that MOVA is in control for the stream of MOVA under test.

- Toggle detection to create demands for phases in each stage of every MOVA stream of the configuration loaded. Verify all configured MOVA force bits (including TO and HI) operate as expected to service all the demands.

- Verify the correct status is shown for the force bits on the Online Data webpage and cross reference with the Controller Status | IO | Virtual bits webpage.

Tip: Open the two webpages on different browser instances and tile side by side.

#### Status Indicator Sliders

There are three status toggle-switch indicators to check underneath the force bits status:

- MOVA Enabled

- On-Control

- CRB

These are read only and should reflect the state of the CRB bit set by the controller, MOVA Enabled and On-control flags set on the Dataset Manager |Settings tab or by MOVA Tools.

- For the stream 1, check the status of the two flags and CRB and verify they change state accordingly when switching signals OFF/ON and by changing modes on the controller to influence the CRB and use the webpages to set/clear the state of the flags.

- Repeat step 1 for the remaining streams.

Important: As of MOVA 8.7RC2, the CRB is not correctly reported by MOVA until after the dataset has been warmed up. This is a kernel issue reported to TRL. Once MOVA has assumed control, it should be correctly reported.

#### Status Indicator Counters

There are two further indicators formatted as counters:

- Error count

- Warmup

These are read only and in normal use, the Error count should be zero and the Warmup should be one more than the number of configured stages. The Warmup starts at zero and increments by 1 every time a stage change is seen. When the value reaches the number of stages + 1, MOVA will assume control provided CRB and MOVA Enabled are ON.

##### Warmup

- With signals OFF, in stream 1, verify the Warmup counter resets to 0 when MOVA Enabled is toggled OFF to disable MOVA.

- Cycle the controller stages and verify the Warmup counts to the number of stages + 1.

- Enable MOVA and toggle one more stage change, observing that the Warmup count remains at number of stages + 1.

- Switch signals ON and verify the Warmup counter decrements by 1. When the controller exits startup and confirms stage 1, the Warmup counter should increment by 1 and MOVA will assume control.

- Switch signals OFF and verify the Warmup counter remains at number of stages +1.

- Finally Disable MOVA and verify the Warmup counter resets to 0

- Repeat steps 1 to 8 for the remaining MOVA streams.

##### Error Count

This has already been tested in section 3.5.1.5

Once Section 3.5.2 has been successfully completed sign off the appropriate sections of the test document.

### Webpage Errors

This has already been tested in section 3.5.1.5

Once Section 3.5.3 has been successfully completed sign off the appropriate sections of the test document.

### Webpage Active Dataset

Uses configurations A,B,C,D, E and F.

Note: This section should be repeated using a range of configurations and datasets.

Important: This test requires knowledge of MOVA and how to navigate MOVA Tools to cross-reference the data presented.

- Load a configuration and associated dataset.

- Navigate to the first MOVA stream: MOVA | Stream 1.

- Use the TRL menu to navigate to Active Data set to see a printout of the current dataset.

- Use TRL MOVA Tools to cross reference the dataset with the MOVA Tools file provided.

- Repeat steps 3 and 4 for each remaining stream of MOVA.

- Repeat steps 1 to 5 using a range of configurations.

Once Section 3.5.4 has been successfully completed sign off the appropriate sections of the test document.

### Webpage TMA Logs

Uses configurations A,B,C,D, E and F.

The principle of this test is to prove the functionality of the Traffic Management Act (TMA) log download facility, not to check the content of the logs for accuracy of data.

Note: This section should be repeated using a range of configurations and datasets.

#### Download

- Load a configuration and associated dataset.

- Navigate to the first MOVA stream: MOVA | Stream 1.

- Use the TRL menu to navigate to TMA Logs. This will display a Download button for that stream.

- Click the button to download the TMA logs. Verify the webpage downloads a file called tma_logs.csv which can be opened in Excel.

#### TMA Logs Structure

- The level of data presented in the log will vary according to how long the dataset has been loaded for and what the tester has done with that dataset during testing.

- Check that the following logs are presented in the CSV file, noting some may contain no data:

  - Flow

  - Occupancy

  - Stage Count and Stage Length

  - Pedestrian Level of Service

  - End of Saturation

  - Oversaturation

  - Faulty Detectors

- Repeat tests 3.5.5.1 and 3.5.5.2 for each MOVA stream in a range of configurations.

Once Section 3.5.5 has been successfully completed sign off the appropriate sections of the test document.

### Webpage SatFlow Logs

Uses configurations A,B,C,D, E and F.

The principle of this test is to prove the functionality of the Saturation Flow logs export facility, not to check the content of the logs for accuracy of data.

Note: This section should be repeated using a range of configurations and datasets.

#### Export

- Load a configuration and associated dataset.

- Navigate to the first MOVA stream: MOVA | Stream 1.

- Use the TRL menu to navigate to SatFlow Logs. This will display Refresh and Export to CSV buttons.

- Click the Refresh button to verify the log is refreshed. It should reappear after a few seconds.

- Click the Export to CSV button to export the log. Verify webpage downloads a file called SatFlowYYYY-MM-DDThh mm ss.mmmX.csv where:

  - YYYY = Year,

  - MM = Month as numeric value,

  - DD = Day as numeric value,

  - hh = hour (24-hour clock format),

  - mm = minutes,

  - ss = seconds,

  - mmm = milliseconds,

  - X = Z or A for GMT or BST.

For example: 2023-11-07T15 59 18.578Z.csv which can be opened in Excel.

#### SatFlow Logs Structure

- Open the file and verify it contains the same data as last displayed on the webpage, noting that some of the data could have been updated.

- Repeat tests 3.5.6.1 and 3.5.6.2 for each MOVA stream in a range of configurations.

Once Section 3.5.6 has been successfully completed sign off the appropriate sections of the test document.

### Webpage MOVA Messages

Uses configurations A,B,C,D, E and F.

The principle of this test is to prove the display of the MOVA messages, not to check the content for accuracy of data.

Note: This section should be repeated using a range of configurations and datasets.

#### Message Display

- Load a config and dataset, and gain MOVA control in all streams.

- Use the TRL menu to navigate to Messages. Initially blank, this will display a continually updating set of MOVA messages, once detectors are toggled.

- Toggle the detectors in a sensible order to reflect traffic flow and monitor the message output.

- Auto-scroll should be ON and SF Message should be OFF by default. The output should be similar to that shown below with the latest message displayed on the last line and always visible. Older messages should move up the window and disappear off the top.

Figure 5 – MOVA Messages (Default View)

- Toggle Auto-scroll OFF, continue to toggle detection, and verify the messages continue but they don’t appear to move up the window. Note that the scroll bar changes to indicate messages further down out of view. Use the scroll bar or wheel mouse to scroll to the bottom of the list of messages.

- Toggle SF Message ON to switch on SatFlow messages. Continue to toggle the detection, on the same approach to ramp up the flow rate and then an SF message should appear after the smooth cycle message SMCYC:

Figure 6 – Example Saturation Flow Message

- Toggle SF Message OFF and verify no more SF messages appear after the SMCYC messages.

- Repeat steps 1 to 7 for each MOVA stream in a range of configurations.

Once Section 3.5.7 has been successfully completed sign off the appropriate sections of the test document.

## MOVA Tools Testing

### Introduction

This section tests the controller connectivity and compatibility with the TRL MOVA Tools program. It is not a functional test of the MOVA Tools software and is limited to operation when a controller is connected.

Tip: This section can be tested in parallel with Section 3.5 Optima Webpage Testing.

Important! Ensure the latest version of MOVA Tools is installed before continuing. This section of testing should be undertaken whenever a new version of MOVA Tools is released by TRL. A good working knowledge of MOVA Tools software operation is expected.

Note: Streams 1 - 4 port numbers are 6030 – 6033 respectively.

### Dataset Management

Uses configurations A,B,C,D and E

Note: This section should be repeated using a range of configurations and datasets.

#### Transfer Dataset

- Set up the controller with a configuration and load the dataset into MOVA Tools

- Choose the stream under test from the Connected to… dropdown list and enter the appropriate IP address and port number for the stream.

- Click Continue followed by Transfer to transfer the dataset and verify the status bar at the bottom shows that the dataset transferred successfully.

- Operate the controller to verify the dataset works correctly, and cross reference with the controller webpages as necessary.

#### Retrieve Dataset

- If continuing from previous test, steps 2 and 3 can be skipped.

- Transfer a dataset matching the controller configuration into the controller and verify it transferred correctly and operates.

- Connect to the appropriate stream under test.

- Click the Dataset Manager (double red/green arrow icon) and change to the Retrieve Tab and click Retrieve dataset. MOVA Tools should inform where the dataset will be saved and invite the user to open the dataset in MOVA Tools.

- Verify the status bar indicates that the dataset file has been retrieved and that it has been saved.

- Compare the saved dataset to the original using MOVA Tools.

Once Section 3.6.2 has been successfully completed sign off the appropriate sections of the test document.

### Junction Mimic Diagram

- Whilst connected to the dataset open the Junction Mimic Diagram and verify it is operating correctly for the running configuration and dataset.

- Verify the following operate as expected:

  - Detectors status,

  - Force bits status,

  - Confirm bits status,

  - CRB status,

  - MOVA Enabled status,

  - On Control status,

  - Demanded Stage,

  - Error Count (see section 3.5.1.5),

  - Warmup Status,

  - Kernel version,

  - Emergency/Priority link status.

Figure 7 – Junction Mimic Diagram Status Indicators

#### Junction Mimic Diagram Display Options

The bottom of the Junction Mimic Diagram has several display options:

Figure 8 – Junction Mimic Diagram Display and Management options

- Verify the correct operation of toggling on/off the Force and Confirm bits.

- Confirm Lane Endsat Colours key can be toggled on/off.

- Check TMA Logs display and can be exported.

- Check SatFlow Logs display and can be exported.

- Check Error Log displays and can be exported (follow steps in 3.5.1.5 to generate errors).

- Check Flows displays and can be exported.

- Check the MOVA Msgs displays and can be exported / logged.

  - Verify Auto Scroll and Show Sat Flow Messages work as expected.

- Check Signal Cycle Diagram functions and the history slider and resume button function.

- Check Raw Det/Output displays detector channels and Output channels. The output channels reflect the data plan in use; refer to section 3.6.4 for changing data plans.

#### Junction Mimic Diagram Runtime Settings

Only Time settings and Operation Management are considered in this test specification.

- On the Time settings tab, verify the time and date can be changed and check against the controller clock.

- Change across daylight savings dates in both directions and check against the controller clock.

- Switch to the Operation Management tab. With MOVA offline, verify all stages can be forced, and the Cancel Current Force button functions as expected.

- Check that the MOVA Enabled and On-Control flags can be correctly set and cleared. Refer to section 3.5.1.4.

- Check the Reset Error Count button functions correctly. Refer to section 3.5.1.5.

Once Section 3.6.3 has been successfully completed sign off the appropriate sections of the test document.

### MOVA Data Plan Switching

Uses configurations A,B,C,D,E and F.

Note: This section should be repeated using a range of configurations and datasets.

#### Overview

MOVA 8 datasets can have up to four data plans, and always have at least one. The dataset is set up as data plan 1, and then data plan 1 can be copied into subsequent data plans where changes are made as required for various anticipated scenarios.

Data plans are switched by timetable and/or trigger detector. For a data plan to function in MOVA, it must be defined to run for at least 15 minutes at some point in the week by timetable. One data plan must be defined as the reversionary plan.

If detector switching is enabled, that data plan can be switched to the active plan by presence of a trigger detector that must stay active to hold the triggered plan. Detector switching overrides the timetabled plans. If MOVA detects a significant jump in controller time of greater than one minute, it will trigger a return to the timetabled data plan even if a data plan detector is active.

Important! The data plan is only ever changed the next time stage 1 runs, whatever the source of the data plan change.

Note: MOVA does not need to be in control to see the data plan changes take effect.

#### Data plan Monitoring (for testing purposes)

Each data plan has identical special conditioning which activates an Output Channel whenever link 1 is red or green. Output Channel 1 is used for data plan 1, up to Output Channel 4 for data plan 4. In addition, configurations A and B have unique detectors in each stream set up to reflect which data plan is functioning so from any stream the data plan in use on any other stream can be determined.

#### Time of Day Operation

- Load the configuration and dataset and connect to controller stream 1 using MOVA Tools.

- MOVA does not have to be in control to check data plan operation.

- Expand the Data Plans tree in Data Outline and note the reversionary plan and timetable settings for the other plans.

- Check the active data plan against the current controller date and time:

  - Active data plan given by Output Channel (all configurations)

  - Active data plan given by detector output (configurations A and B)

  - Active data plan given by Dataset Manager | Activation tab, clicking the refresh button.

- Repeat step 4 in the remaining MOVA streams of configuration under test.

- From Runtime Settings, change the MOVA time to check the non-reversionary plans operate in line with the timetable, remembering that the change occurs when stage 1 commences. Allow the controller to cycle round at least twice to show that the running data plan remains active for more than one cycle.

- Repeat step 6 in the remaining MOVA streams.

#### Reversionary Plan

- Load the configuration and dataset and connect to controller stream 1 using MOVA Tools.

- MOVA does not have to be in control to check data plan operation.

- Expand the Data Plans tree in Data Outline and change the reversionary plan to a different data plan and adjust the timetable for the remaining plans accordingly.

- Test the timetable operation in accordance with test 3.6.4.3

- Repeat the test in the remaining MOVA streams.

#### Data Plan Detector Triggering

This test should be done with MOVA offline and again with MOVA online.

- Load the configuration and dataset and connect to controller stream 1 using MOVA Tools.

- Expand the Data Plans tree in Data Outline to note the MOVA detectors required to trigger each data plan. Cross check with the controller configuration paperwork to ascertain the required inputs to activate those detectors.

- Test each detector activates the correct data plan, and that it keeps that data plan active across multiple cycles through stage 1.

- Check all the methods to read the active data plan (step 4 in test 3.6.4.3) agree with each other.

- Check the detector triggering works overrides all timetabled data plans including the reversionary plan.

- Repeat the test in the remaining MOVA streams.

#### Clock Change Behaviour: Changes < 1 Minute

This test should be done with MOVA offline and again with MOVA online.

- Load the configuration and dataset and connect to controller stream 1 using MOVA Tools.

- On the Time settings tab and change the time of day by less than 1 minute from the current time.

- Verify this does not change the running data plan called by timetable. Run the controller through more than one appearance of stage 1.

- Repeat steps 4 and 5 for all the timetabled data plans.

- Activate a data plan by detector triggering and confirm change to that data plan.

- Repeat step 4 and verify this does not cause a change back to the timetabled plan. Run the controller through more than one appearance of stage 1.

- Repeat step 8 for all data plans when triggered by detector.

- Repeat the test in the remaining MOVA streams.

#### Clock Change Behaviour: Changes > 1 Minute

This test should be done with MOVA offline and again with MOVA online.

- Load the configuration and dataset and connect to controller stream 1 using MOVA Tools.

- On the Time settings tab and change the time of day by more than 1 minute from the current time.

- Verify the running data plan called by timetable updates to reflect the current day/time the first time stage 1 appears. Run the controller through more than one appearance of stage 1 to verify it does not change again.

- Repeat steps 2 and 3 for all the timetabled data plans.

- Trigger a different data plan to the current one by detector and wait for it to take effect.

- On the Time settings tab and change the time of day by more than 1 minute from the current time.

- Verify this causes the controller to return to the timetabled data plan the next time stage 1 appears.

- Run the controller through more than one appearance of stage 1 and verify the controller returns to running the detector triggered data plan.

- Run the controller through more than one appearance of stage 1 to show that the detector triggered data plan remains the active plan now the clock is stable.

- Repeat steps 5 to 9 for all the detector triggers with different combinations of timetabled plan, not just one timetabled plan or just the reversionary plan.

- Repeat the test in the remaining MOVA streams.

#### Removing Time of Day information

Uses configuration B.

This test should be done with MOVA offline and again with MOVA online.

- Drive the controller to stage 2 and change the time of day to 00:12.

- Drive the controller to stage 1 and verify the data plan changes to 2 on appearance of stage 1 confirm.

- Drive the controller to stage 2, activate detector 64 to demand data plan 4.

- Drive the controller to stage 1 and verify the active data plan changes to 4.

- Drive the controller to stage 2, and then back to stage 1 a few times to prove that data plan 4 remains active.

- Drive the controller to stage 2, remove detector 64 and drive back to stage 1.

- Verify the active data plan changes to data plan 2 as requested by the timetable.

- By now the controller time should be approaching 00:15 so drive the controller to stage 2 and wait for the time to roll over 00:15.

- Drive the controller to stage 1 and verify the active data plan changes to data plan 1 as requested by the timetable.

- Repeat steps 1 to 5.

- Click on the Dataset Manager, select the Time of Day tab and click Remove Time of Day.

- Repeat step 6.

- Verify data plan 4 remains active, ignoring the timetable.

- Wait for the controller time to roll over 00:15 and verify the data plan does not change to data plan 1 as requested by the timetable but remains in data plan 4.

- Drive the controller to stage 2, activate detector 61 then remove detector 64.

- Drive the controller to stage 1 and verify data plan 1 becomes active.

- Drive the controller to stage 2, and then back to stage 1 a few times to prove that data plan 1 remains active.

- Drive the controller to stage 2, activate detector 64 then remove detector 61.

- Drive the controller to stage 1 and verify the active data plan changes to 4.

- Drive the controller to stage 2, and then back to stage 1 a few times to prove that data plan 4 remains active.

- Drive the controller to stage 2, remove detector 64 and drive back to stage 1.

- Verify the active data plan changes to data plan 1 as requested by the timetable.

- Drive the controller to stage 2, and then back to stage 1 a few times to prove that data plan 1 remains active.

- Repeat the steps 1 to 24 with MOVA online.

- Repeat the test for remaining streams of MOVA.

Once Section 3.6.4 has been successfully completed sign off the appropriate sections of the test document.

## MOVA Special Conditioning Testing

Uses configurations A,B,C,D and E.

All the datasets used in the test specification contain MOVA Special Conditioning which is used to switch detectors and output channels to show which data plan is in use. This was done to facilitate the data plan switching testing.

Note: This section should be repeated using a range of configurations and datasets.

### Output to User Variable

This has been already tested as it forms part of the special conditioning to test the data plan switching.

### Output to Output Channel

This has been already tested as it forms part of the special conditioning to test the data plan switching.

### Output to Detector

Each stream of configuration A and B has special conditioning Rule 4 which is a copy of Rule 3 that sets Output Channels 1 to 4 for data plans 1 to 4. Instead of setting the output channels, Rule 4 sets detectors 17 to 20 for active data plans 1 to 4 respectively.

Important! MOVA Tools displays only the Raw Detector data, not the processed detector data, therefore detectors processed by MOVA Special conditioning only show on the TRL Online Data page in the controller webpages.

- Load configuration A and the associated MOVA dataset

- Check that for each data plan, the corresponding detector is activated when a data plan is active.

- Repeat step 2 for each stream of MOVA configured.

- Repeat steps 1 to 3 for configuration B.

### Power Cycle of Controller

Note: This test should be done with MOVA offline and again with MOVA online, with and without an active connection to MOVA tools.

- Load the configuration and dataset and switch signals ON.

- Verify the MOVA special conditioning is operating as expected by triggering a data plan change and observing the Output Channels / Detectors change on the TRL webpages.

- Power cycle the controller and wait for MOVA to re-establish communications with the webpages.

- Repeat step 2.

- Connect to controller stream 1 via MOVA Tools and repeat step 2, this time checking the output channels from the MOVA mimic diagram Raw detector / Channels window.

- Repeat steps 1 to 5 with MOVA in control.

- Repeat the test for each configured stream of MOVA.

### Cross-Stream Detector Activation from Output Channels

Uses configurations A and B

Configurations A and B have all 8 MOVA output channels for each stream (including to same stream) mapped as follows:

| Detector | Output Channel | Comment |
| --- | --- | --- |
| 21 | M1 OC1 | Stream 1 Data Plan 1 |
| 22 | M1 OC2 | Stream 1 Data Plan 2 |
| 23 | M1 OC3 | Stream 1 Data Plan 3 |
| 24 | M1 OC4 | Stream 1 Data Plan 4 |
| 25 | M1 OC5 |  |
| 26 | M1 OC6 |  |
| 27 | M1 OC7 |  |
| 28 | M1 OC8 |  |
| 31 | M2 OC1 | Stream 2 Data Plan 1 |
| 32 | M2 OC2 | Stream 2 Data Plan 2 |
| 33 | M2 OC3 | Stream 2 Data Plan 3 |
| 34 | M2 OC4 | Stream 2 Data Plan 4 |
| 35 | M2 OC5 |  |
| 36 | M2 OC6 |  |
| 37 | M2 OC7 |  |
| 38 | M2 OC8 |  |
| 41 | M3 OC1 | Stream 3 Data Plan 1 |
| 42 | M3 OC2 | Stream 3 Data Plan 2 |
| 43 | M3 OC3 | Stream 3 Data Plan 3 |
| 44 | M3 OC4 | Stream 3 Data Plan 4 |
| 45 | M3 OC5 |  |
| 46 | M3 OC6 |  |
| 47 | M3 OC7 |  |
| 48 | M3 OC8 |  |
| 51 | M4 OC1 | Stream 4 Data Plan 1 |
| 52 | M4 OC2 | Stream 4 Data Plan 2 |
| 53 | M4 OC3 | Stream 4 Data Plan 3 |
| 54 | M4 OC4 | Stream 4 Data Plan 4 |
| 55 | M4 OC5 |  |
| 56 | M4 OC6 |  |
| 57 | M4 OC7 |  |
| 58 | M4 OC8 |  |

- Verify the Output Channels as configured above function correctly in each data plan and stream:

  - The first 4 channels of each stream have been tested already as part of the data plan switching tests.

  - Modify or copy the existing special conditioning to test outputs 5-8 in each stream function.

- Repeat the test using configuration B.

Once Section 3.7 has been successfully completed sign off the appropriate sections of the test document.

## Miscellaneous Tests

Uses configurations A,B,C,D and E.

Note: These tests may have been observed during the testing of other parts of the specification.

### CRB Operation

This may have already been observed in previous tests.

- Verify a mode change or MOVA inhibit button drops the CRB for stream 1 gracefully and MOVA regains control when mode allows.

- Repeat for remaining streams of MOVA.

### Minor Configuration Change

- Load a configuration and dataset and verify MOVA operation.

- Make a change to the configuration, adding a new VA detector for a phase, do not add to the MOVA detection screen however, up-issue the version to the next major number.

- Switch signals OFF, upload the configuration to the controller and make active.

- Verify the MOVA dataset is unaffected by the change and can still take control.

### Major Configuration Change

Note: This test may have already been observed while changing configurations and datasets.

- Load a configuration and dataset and verify MOVA operation.

- Change the configuration for one of the others used in this test specification.

- Without updating the MOVA dataset, visit the TRL webpages and verify MOVA is still functioning (not crashed) but may be offline if the dataset is very different from the one designed for the current configuration.

- Check across a range of streams, configurations, and datasets.

### MOVA Emergency/Priority Links Operation

Uses configurations C,D and E.

- Verify the operation of the MOVA emergency/priority Links within the datasets. Check that links are called/extended as appropriate.

### Connection to Controller Lost Warning

- Ensure that the controller MOVA webpages are working as normal.

- Visit the Online Data page and establish that the page is displaying correctly.

- Remove the ethernet cable or power off the controller and check:

  - Around 5 – 10 seconds after power off, the MOVA time changes to red indicating connection lost,

  - Around 30 seconds after power off, the message Warning: connection to controller has been lost is displayed.

- Check the remaining streams.

Once Section 3.8 has been successfully completed sign off the appropriate sections of the test document.

# Document Control

## Maintenance and Distribution

This document is subject to formal change and control procedures as required by the Quality Management System (QMS).

## Amendment History

| Issue | Date | Change Descriptions | Author |
| --- | --- | --- | --- |
| Issue 1 | 27/11/2023 | Initial draft for internal review | Iain Ross |

## Abbreviations

CRB	Controller Ready Bit

ELV	Extra Low Voltage

Fn		Force Stage n (bit)

HI		Hurry Inhibit (bit)

I/O		Input / Output

iMOVA	Integral MOVA

LED	Light Emitting Diode

MOVA	Microprocessor Optimised Vehicle Actuation

PC		Personal Computer

SCB	Special Conditioning Bit

TMA	Traffic Management Act

TO		Take Over (bit)

TRL	Transport Research Laboratory

USB	Universal Serial Bus

UCM	Universal Content Management

VA		Vehicle Actuated

## Referenced Documents

| Title | Doc Ref | Issue |
| --- | --- | --- |

## Related Documents

| Title | Doc Ref | Issue |
| --- | --- | --- |
| Optima Integral MOVA Approval | 503223 | Latest |
| Optima iMOVA Test Results | 672664 | Latest |
| Optima iMOVA Functional Testing Configurations | 673512 | Latest |
| Optima Functional Test Specification | 239467 | Latest |
| Optima Functional Test Results | 272451 | Latest |